DIGITIZE

VI.0-003

August 22, 1989

The DIGITIZE program will transfer a drawing to a host computer, resulting in a

VG input file.  This is done by "digitizing" the drawing, using a Tektronix

4957 tablet attached to a Tektronix terminal.  The output file produced by the

program, consisting of VG MOVE and DRAW commands, will re-create the drawing

you have DIGITIZEd.

Setting Up the Tablet

---------------------

If it has not already been done, install the tablet as follows:

1.  Connect the 4-button cursor, or "puck", to the tablet.

2.  Connect the tablet's communication cable to port 0 of the corresponding

terminal.

3.  Connect the tablet's power supply to the tablet.

4.  Connect the power supply to a wall power outlet.

For help in setting up or operating the tablet, see the Tektronix 4957 Graphics

Tablet User's Manual, or the appropriate systems person.

The program will support any size paper that fits on the tablet; DIGITIZE

always scales to the VG page dimensions of 14 x 11 inches.  Put the paper with

the picture to be digitized on the tablet, as close to the bottom edge as

possible (depending on the size of the paper).  This is necessary due to the

fact that the top 2.5 inches of the tablet are not part of the active area for

the DIGITIZE program (points digitized in this area will not appear on the

terminal screen).  It is recommended that you tape the picture to the tablet,

so the paper will not move as you are digitizing.

Starting the Program

--------------------

Log on to the VAX and start the DIGITIZE program by typing

$ DIGITIZE

In order to translate tablet coordinates to VG coordinates, DIGITIZE needs to

know the tablet coordinates of the lower, left and upper, right corners of the

paper.  The program will prompt you to "Please click the lower, left corner of

the page"; simply place the puck on lower, left corner of the paper on the

tablet, then press any one of the buttons on the puck.  The program will then

prompt for the opposite corner - click this corner in the same manner.  The

corners will be marked with an "X" on the screen; the area within these points

represents the active working area for digitizing.

You are now ready to begin digitizing the drawing.

Digitizing the picture

----------------------

"Digitizing" consists of tracing the outline of a drawing by locating points at

the ends of line segments.  This is done by placing the puck on top of a point,

positioning the crosshairs to the desired position, and pressing a button

corresponding to either a MOVE or a DRAW command.  When a button is pressed on

the puck, DIGITIZE reads and stores the X,Y coordinates of the point from the

tablet, translates them to VG coordinates, and creates the corresponding VG

command.  (For more information about the VG program, see the VG Manual

(Automation Manual, Appendix D.)  The default cursor position on startup is the

lower, left corner of the screen "clicked off" at the beginning of the program

(0,0 in VG coordinates).  Erroneously entered points may be deleted using the

DELETE command. When all desired points have been entered, give the EXIT

command, and the VG file will automatically be created.  The above mentioned

commands are described below.  Note that pressing invalid keys on the keyboard

or buttons on the puck will not have any effect on the program.  For more

information about the VG program, see the VG Manual (Automation Manual,

Appendix D).

- ARC

The ARC command would generate a VG ARC command.  To draw an arc, it is

necessary to specify three points.  For the ARC command, the first point is the

current point; therefore it is necessary to click off two more points. A

circular arc will be drawn from the current position through these two points.

A circle will be drawn if the second point specified is equal to the starting

position of the arc.  Use the GREEN button on the puck to specify ARC

coordinates. (This command has not yet been implemented.)

- BELL

The BELL command causes the terminal's bell to sound whenever a MOVE or DRAW

command is issued.  The bell tone for a MOVE command is slightly longer than

that of a DRAW; this is done to enable the user to "hear" when he

unintentionally enters a MOVE instead of a DRAW or vice-versa. This command is

an on/off type command; i.e., giving the BELL command once turns on the bell;

giving it again turns it off. The default is off.  Turn the bell on (and off)

by typing "B" on the keyboard.

- COMMENT

The COMMENT command allows you to enter a VG COMMENT into the output file. Type

"C" on the keyboard, and you will be prompted to "Type a comment - 80

characters maximum - <CR> to end:"; type the comment and press the <Return>

key. The COMMENT will be placed in the file after the last command given.  Note

that COMMENTs are UN/DELETEd in sequence the same as MOVE and DRAW commands.

- DELETE/UNDELETE commands

The DELETE command will erase the last entered MOVE or DRAW command.  To use

this command, type the letter "D" (or "d") ON THE KEYBOARD (it's not necessary

to press the RETURN key).  The screen will be redrawn to reflect the DELETEd

command.  UNDELETE will restore what was just DELETEd.  Type "U" ON THE

KEYBOARD.  The DELETEd point, line or arc will be redrawn.  Note that the

DELETE/UNDELETE commands are many for one, that is, only the LATEST point in a

series of DELETE commands will be UNDELETEd.

- DRAW command

The DRAW command generates a VG DRAW command.  Specify the X,Y coordinates to

DRAW to by positioning the puck crosshairs on the desired location and press

the WHITE button. The VG coordinates of this point will be displayed on the

screen. A series of DRAW commands may be given after a MOVE command; this

sequence would be useful, for instance, to trace the outer edge of a drawing

(see the Simple Example, below).

- EXIT

The EXIT command, naturally, terminates execution of the program.  The VG input

file is created at this point.  To EXIT, type the letter "E" (or "e") ON THE

KEYBOARD (as for DELETE, there's no need for a <Return>).  The VG input file

will be called DIGITIZE.VG.  This file will be all set up to be run through

VG; the device type will be '4115'.

- HELP

The HELP command will display a help message consisting of a short description

of each DIGITIZE command.  To receive HELP, type "H" ON THE KEYBOARD.

- MOVE command

The DIGITIZE MOVE command generates a VG MOVE command for the output file. This

command would usually be the first one given when starting to digitize a

drawing (as would be done for a regular VG file to locate the starting position

for the cursor).  Specify the the X,Y coordinates to MOVE to by positioning the

puck crosshairs on the desired position on the drawing being digitized and

press the YELLOW button on the puck. The (VG) coordinates of this point will be

displayed on the terminal screen after the button is pressed.

A Simple Example

----------------

Suppose you had the simple rectangle below on an 8.5 x 11 inch paper:

+-----------------+

|                 |

|                 |

|                 |

+-----------------+

To create a VG file with commands to recreate this rectangle by using DIGITIZE,

first follow the steps above to set up the tablet.  Then, start the DIGITIZE

program and click off the corners (also described above).

To digitize the rectangle, you could start by placing the cursor on the lower,

left corner ("+"), and pressing the YELLOW button on the puck.  This generates

a VG MOVE command. Next, place the puck on the upper, left corner and press the

WHITE button.  This generates a VG DRAW command which will DRAW the line

between the two points you just "digitized". To finish the rectangle,

successively place the puck on the upper right, lower right, and lower left

corners, pushing the WHITE button at each location. Finally, exit the program

by typing "E", and VG the file DIGITIZE.VG.  A rectangle similar to the one you

just created using DIGITIZE should appear on the screen.