SUBROUTINE	USRDB$DUMP_OPEN( NEWOLD )

PURPOSE:  OPEN THE USER DATABASE FORMATTED DUMP FILE.  DEPENDING UPON SWITCH

PASSED, IT IS OPENED FOR READ ONLY OR FOR WRITE.

ARGUMENT:	TYPE:		DESCRIPTION:

=========	=======		======================================

IN	NEWOLD		INT*4		SWITCH VARIABLE FOR HOW THE DUMP

FILE IS TO BE OPENED.

SUBROUTINE	USRDB$SYSTEM_OPEN( )

PURPOSE:  OPEN THE USER DATABASE DATA FILE FOR READ & WRITE OPERATIONS.

ALSO OPENS THE TRANSLATION DATA FILE FOR READ OPERATIONS.

NO ARGUMENTS PASSED TO THIS ROUTINE.

SUBROUTINE	USRDB$USER_OPEN( )

PURPOSE:  OPEN THE USER DATABASE DATA FILE FOR READ ONLY OPERATIONS.

ALSO OPENS THE TRANSLATION DATA FILE FOR READ OPERATIONS.

NO ARGUMENTS PASSED TO THIS ROUTINE.

SUBROUTINE	USRDB$CLOSE_DB( )

PURPOSE:  CLOSES THE USER DATABASE DATA FILE AND THE TRANSLATION DATA FILE.

NO ARGUMENTS PASSED TO THIS ROUTINE.

SUBROUTINE	USRDB$CLOSE_DUMP( )

PURPOSE:  CLOSE THE USER DATABASE FORMATTED DUMP FILE.

NO ARGUMENTS PASSED TO THIS ROUTINE.

FUNCTION	USRDB$GET_INIT( USRDB$REC, SORT_KEY, MATCH_STRING, MATCH_KEY )

PURPOSE:  INITIAL INDEXED READ OF THE USER DATABASE DATA FILE.  MUST BE

PERFORMED PRIOR TO EXECUTING THE USRDB$GET_RECORD.  RETURNS

FIRST RECORD OF THE DATABASE BASED UPON CRITERIA PROVIDED.

ARGUMENT:	TYPE:		DESCRIPTION:

=========	=======		======================================

RET	USRDB$GET_INIT	INT*4		VALUE RETURNED DEPENDS UPON IF A RECORD

WAS FOUND WHICH MATCHED THE SPECIFIED

CRITERIA OF THE USRDB$GET_INIT ACTIVITY.

RET	USRDB$REC	CHR*1024	RECORD READ FROM THE USER DATABASE FILE.

IN	SORT_KEY	INT*4		THE INDEXED FIELD TO BE USED FOR MATCHING.

THE SORT_KEYS ARE PASSED USING PARAMETERS

DEFINED IN THE FILE LIFE_DEV:DB_PARAMS.INC.

IN	MATCH_STRING	CHR*(*)		STRING TO MATCH, BASED UPON THE TYPE OF

MATCH SPECIFIED.

IN	MATCH_KEY	INT*4		TYPE OF MATCH TO PROVIDE AGAINST STRING.

MATCH KEYS ARE PASSED USING PARAMETERS

DEFINED IN THE FILE LIFE_DEV:DB_PARAMS.INC.

FUNCTION	USRDB$GET_RECORD( USRDB$REC )

PURPOSE:  USED TO READ REMAINING RECORDS OF THE USER DATABASE DATA FILE.

AFTER THE INITIAL READ PERFORMED BY THE USRDB$GET_INIT FUNCTION,

THIS FUNCTION PERFORMS SEQUENTIAL READS OF THE DATABASE BASED

UPON THE INITIAL CRITERIA DEFINED BY THE USRDB$GET_INIT ROUTINE.

ARGUMENT:	TYPE:		DESCRIPTION:

=========	=======		======================================

RET	USRDB$GET_INIT	INT*4		VALUE RETURNED DEPENDS UPON IF A RECORD

WAS READ FROM THE DATABASE FILE.

RET	USRDB$REC	CHR*1024	RECORD READ FROM THE USER DATABASE FILE.

FUNCTION	USRDB$PUT_RECORD( USRDB$REC )

PURPOSE:  WRITES A NEW RECORD TO THE USER DATABASE DATA FILE.

ARGUMENT:	TYPE:		DESCRIPTION:

=========	=======		======================================

RET	USRDB$PUT_RECORD  INT*4		VALUE RETURNED DEPENDS UPON IF A RECORD

WAS WRITTEN TO THE USER DATABASE.

IN	USRDB$REC	CHR*1024	RECORD TO BE WRITTEN TO THE USER DATABASE.

FUNCTION	USRDB$UPDATE_RECORD( USRDB$REC )

PURPOSE:  UPDATE A RECORD CONTAINED IN THE USER DATABASE DATA FILE.

ARGUMENT:	TYPE:		DESCRIPTION:

=========	=======		======================================

RET	USRDB$UPDATE_RECORD

INT*4		VALUE RETURNED DEPENDS UPON IF A RECORD

WAS WRITTEN TO THE USER DATABASE.

IN	USRDB$REC	CHR*1024	RECORD TO BE WRITTEN TO THE USER DATABASE.

FUNCTION	USRDB$DELETE_RECORD( USRDB$REC )

PURPOSE:  TO DELETE A RECORD FROM THE USER DATABASE DATA FILE.

ARGUMENT:	TYPE:		DESCRIPTION:

=========	=======		======================================

RET	USRDB$DELETE_RECORD

INT*4		VALUE RETURNED DEPENDS UPON IF A RECORD

WAS DELETED FROM THE USER DATABASE.

IN	USRDB$REC	CHR*1024	RECORD TO BE DELETED FROM THE USER DATABASE.

FUNCTION	USRDB$CLEAR_RECORD( USRDB$REC )

PURPOSE:  TO INITIALIZE THE USRDB$REC BUFFER STRING TO DEFAULT VALUES.

ARGUMENT:	TYPE:		DESCRIPTION:

=========	=======		======================================

RET	USRDB$CLEAR_RECORD

INT*4		VALUE RETURNED ALWAYS SUCCESSFUL.

RET	USRDB$REC	CHR*1024	A BUFFER RECORD WITH DEFAULT VALUES.

PRIM_WORK    = 'P###'		INSTITUTION  = '-SELF-'

COUNTRY      = 'USA'		NETWORK_ID   = 'SL'

NODE_ID      = 'LIFE'		NAME_TITLE   = '-NONE-'

MAIL_STOP    = '-NONE-'		STREET_ADR_1 = '-NONE-'

STREET_ADR_2 = '-NONE-'		PHONE_NMBR   = '-NONE-'

FAX_INFO     = '-NONE-'		OTHER_NET    = 'NA'

NOTIFY       = '0'		SEC_WORK     = 'P###P###P###'

PRIM_ACTIVITY = 'W##'		SEC_ACTIVITY  = 'W##'

PRIN_EMPLOYEE = 'SELF'

ACCESS_CONTROL = '??'		SHORT_INSTITUTE = '-SELF-'

FUNCTION	USRDB$GET_FIELD( USRDB$REC, FIELD_ID, RETURN_STRING, LENGTH )

PURPOSE:  OBTAIN VALUE CONTAINED IN A SPECIFIED FIELD OF THE USER

DATABASE DATA FILE RECORD.

ARGUMENT:	TYPE:		DESCRIPTION:

=========	=======		======================================

RET	USRDB$GET_FIELD	INT*4		SUCCESS/FAILURE FLAG OF THE GET OPERATION.

IN	USRDB$REC	CHR*1024	CURRENT USER DATABASE RECORD.

IN	FIELD_ID	INT*4		THE USER DATABASE FIELD THAT A VALUE IS

BEING REQUESTED FROM THE CURRENT RECORD.

RET	RETURN_STRING	CHR*(*)		THE STRING FOUND IN THE USER DATABASE

FIELD DESIGNATED.

RET	LENGTH		INT*2		LENGTH OF THE STRING FOUND IN THE USER

DATABASE FIELD DESIGNATED.

FUNCTION	USRDB$PUT_FIELD( USRDB$REC, FIELD_ID, NEW_STRING, LENGTH )

PURPOSE:  PLACE A VALUE INTO A SPECIFIED FIELD OF A USER DATABASE

DATA FILE RECORD.

ARGUMENT:	TYPE:		DESCRIPTION:

=========	=======		======================================

RET	USRDB$GET_FIELD	INT*4		SUCCESS/FAILURE FLAG OF THE PUT OPERATION.

IN/RET	USRDB$REC	CHR*1024	CURRENT USER DATABASE RECORD.

IN	FIELD_ID	INT*4		THE USER DATABASE FIELD THAT A VALUE IS

BEING PLACED INTO THE CURRENT RECORD.

RET	RETURN_STRING	CHR*(*)		THE STRING BEING PLACED INTO THE USER

DATABASE FIELD DESIGNATED.

RET	LENGTH		INT*2		LENGTH OF THE STRING BEING PLACED IN THE

USER DATABASE FIELD DESIGNATED.

SUBROUTINE	USRDB$TRANSLATE( IN_STRING, OUT_STRING, ISTAT )

PURPOSE:  THIS ROUTINE IS USED TO OBTAIN THE TRANSLATION OF AN ENCODED

USER DATABASE FIELD FOR THE USE BY OUTPUT PROGRAMS.

THE DATAFILE UTILIZED BY THIS ROUTINE IS OPENED BY THE USRDB$SYSTEM_OPEN

OR BY THE USRDB$USER_OPEN ROUTINES PREVIOUSLY DOCUMENTED.

ARGUMENT:	TYPE:		DESCRIPTION:

=========	=======		======================================

IN	IN_STRING	CHR*(*)		SEARCH STRING FOR AN INDEXED KEY RETRIEVAL.

RET	OUT_STRING	CHR*(*)		VALUE ASSOCIATED WITH SEARCH STRING.

RET	ISTAT		INT*4		STATUS OF THE INDEX FILE READ OPERATION.

FUNCTION	USRDB$PUT_DUMP( USRDB$REC )

PURPOSE:  WRITE A DUMP FILE RECORD FOR AN USER DATABASE ENTRY.

ARGUMENT:	TYPE:		DESCRIPTION:

=========	=======		======================================

RET	USRDB$PUT_DUMP	INT*4		SUCCESS/FAILURE FLAG OF THE PUT OPERATION.

IN	USRDB$REC	CHR*1024	CURRENT USER DATABASE RECORD.

FUNCTION	USRDB$GET_DUMP( USRDB$REC, UPD_GET )

PURPOSE:  READ A DUMP FILE SET OF RECORDS, AND UPDATE A DATABASE RECORD BUFFER.

ARGUMENT:	TYPE:		DESCRIPTION:

=========	=======		======================================

RET	USRDB$PUT_DUMP	INT*4		SUCCESS/FAILURE FLAG OF THE GET OPERATION.

IN/RET	USRDB$REC	CHR*1024	USER DATABASE RECORD CREATED FROM DUMP.

IN	UPD_GET		INT*4		FLAG, DETERMINES IF DUMP RECORD IS

USED TO UPDATE AN EXISTING RECORD OR

IF A NEW DATABASE RECORD IS TO BE

CREATED BY THE FUNCTION.

IF VARIABLE UPD_GET EQUALS PARAMETER USRDB$_BUILD_RECORD, THEN A

NEW RECORD IS CREATED FROM THE DUMP FILE INPUT.

IF VARIABLE UPD_GET EQUALS PARAMETER USRDB$_DUMP_UPDATE, THEN AN

EXISTING RECORD IS UPDATED USING DATA FROM THE DUMP FILE INPUT.

FUNCTION	USRDB$LOCATE( FIELD_ID, POINTER )

PURPOSE:  LOCATE THE POINTER TO THE FIELD_ID PASSED INTO THE ROUTINE.

POINTER CAN THEN BE USED BY THE CALLING ROUTINE TO CROSS

REFERENCE ASSOCIATED DATA TO THE FIELD_ID PASSED.

ARGUMENT:	TYPE:		DESCRIPTION:

=========	=======		======================================

IN	FIELD_ID	INT*4		FIELD IDENTIFICATION OF THE USER

DATABASE FIELD BEING REFERENCED.

RET	POINTER		INT*4		POINTER TO THE LOCATION OF THE

FIELD IDENTICATION WITHIN THE

CROSS REFERENCE DATA ARRAY.