From:	EVMS::TGOODWIN     22-SEP-1995 11:51:13.04

To:	EVERHART

CC:

Subj:	AEN/Target Mode Investigation - First Draft

SCSI Architecture - AEN/Target Mode Investigation

Question: How should we handle AEN/Target Mode ?  Is it best to put

any of it into the port driver (other than interrupt recognition) ?

Should a new class driver be present ?

There are two levels at which the SCSI architecture should provide

support for Target Mode/AEN.  The first level is mandatory because

it is needed to support the current SCSI Clusters functionality and

the SCSI Device Naming project. The second level is optional ( in

that OpenVMS has no current dependency based on this functionality)

but necessary if we are to claim SCSI target mode support.

The first level of support involves receiving both the 'Test Unit

Ready' and 'Inquiry' commands and responding with the proper

response data.  This functionality must be present in the port

driver so that SCSI cluster configuration and SCSI device naming

can function correctly.  The sending and receiving of these

messages must be done prior to the loading of SCSI class drivers

for the devices to be named properly.  I recommend that the code to

generate the response to these messages be part of the port common

code layer so that there will be consistent reponse data for these

messages regardless of the actual adapter involved.  Servicing the

AEN interrupts and delivering the command to the port common layer

becomes part of the port specific layer for each port which will

support SCSI clusters and/or target mode.  This allows us to be

selective of the adapters on which we want to provide this

capability.

The second level of support involves support for the rest of

the mandatory command set for a processor device and any optional

commands from the processor command set which we want to support.

The functionality of this second level is needed if we are going

to use the SCSI bus ( or any future variant of the SCSI bus) as

a network or cluster interconnect.  The support for this level

could be accomplished by creating a processor device class driver

which would establish a procedure callback link to the port driver

(in the port common layer).  All incoming commands which are not

handled by the port driver would be passed to the class driver

through this callback and the class driver would provide the

returned data and status (including any check condition sense data)

to the port driver.

One of the main issue which remains to investigate is whether

this processor class driver would be connected at the adapters

ID and LUN and handle all traffic to other processor devices

on the SCSI bus or whether there would be a separate class driver

instance at the ID and LUN for each target processor device on the

SCSI bus.  The first option is based on the LAN model of one driver

for each network adapter while the second option is based on

the SCSI model with a separate driver for each target device on

the SCSI bus.

My feelings at this point are that the SCSI architecture must

include the first level of AEN/Target mode support and the

design of the necessary port/class interface to support the

second level.  The actual design and coding of the processor

device class driver can wait until the market demands of it

can justify the expense.  Alternatively, the actual processor

device class driver might be developed by a group outside of

OpenVMS engineering (MSC or third-party), but the interface

needs to be clearly designed and documented by OpenVMS.