Error Avoider

This is a software "wedge" system which uses an intercept driver to

do extra error recovery for disks. It was originally found to be useful

for certain optical disk models, since optical disks have very frequent

errors and some were exceeding error limits that a patched DKdriver would

allow. It may also be that there are circumstances where unusual load

conditions or problems with generic disk firmware can produce errors

which may disappear if retried, but which are not retried (or not retried

enough) by VMS disk drivers. Most such errors on normal SCSI disks

indicate impending drive failure, but on some occasions they may indicate

simply firmware which reports too many errors, or media (as in an old

floppy disk) which is approaching end of life, but which can be read with

very large retry counts. For these purposes, this wedge can spell the

difference between being able to use a device and not being able to. It

is a matter for experimentation whether this system will actually help,

but it is easy to install and use, and has no effect on operation save

that, FOR THE PARTICULAR DISK(s) INTERCEPTED, it allows a number

(currently 8) retries on read or write before giving up.

The driver, JGdriver, is an intercept driver furnished to provide

extended error retry for a VMS device (usually a disk, but any device can

be used). Note that this code is directed at disks primarily or at other

similar devices where the UCB does NOT GET DELETED.

The intercept driver will cause erroneous read or write operations on its

target device to be (invisibly) retried up to a settable number of times

(8 as it currently stands...this is assembled into jgdriver) in the event

of error. Its use is otherwise invisible to users and it operates by

intercepting driver start_io and using the IRP$L_PID hook to get access

to the IRP after I/O but before the user app sees it.

The AXP version of JGdriver just uses the IRP space from IRP$Q_QIO_P3 on

through IRP$Q_QIO_P5 (in VMS 7.0 and up) for storage of information about

how many times (if at all) the IRP has been retried, and some bits of

extra information needed to finish it up. Since this information should

by start-io time have been fully encoded into the IRP it is expected this

will not cause problems.

The VAX version uses the IRP$L_KEYDESC field (so long as this is not

already in use) to point to a small block it allocates from pool to hold

this information, because there is not room for it in the VAX IRP.

Operation otherwise is the same. At start-io, for r/w,the intercept code

looks for a magic number in its data storage and if it finds it, assumes

that it has already set the IRP up. If it does not find it, it sets it up

and sets the IRP field IRP$L_PID to cause I/O post to go to a routine

within the intercept driver on completion. It then calls the original

start-io and returns. (If the operation is not read or write, it just

calls the original start-io without touching the IRP.)

The completion code examines the I/O status about to be returned to the

user. If the low bit is 1 (success) it just uses com$post to postprocess

the IRP (since the driver has already called REQCOM) If the low bit is 0,

it decrements the error retry count it has and requeues (via insioqc) the

I/O (from fork level!) to the intercepted driver again. If the retry

count is exhausted it just cleans up and calls com$post (actually, for

axp, the step2 replacement) to return the final status to the user.

This operation has proven handy for some types of disk storage where the

retry operation of the native drivers has been inadequate. In particular,

the VAX DKdriver is not as careful as it might be to retry potentially

retriable operations, so that using this intercept provides an invisible

yet highly effective way of ensuring that an error is repeatable before

the user code sees it. Where timing or other transient conditions cause

problems, this gives a large advantage. On optical media it has been

dramatic on VAX. The Alpha DKdriver is more careful, but even there, this

driver will ensure that errors due to timeouts under heavy load are

retried before being returned. Since this approach requires NO driver

code changes it is attractive; it can be applied for those disks needing

it, and not applied for others. The intercept is device unit specific,

unlike one which might patch a class driver in memory or something of

that sort. Units not intercepted operate EXACTLY as before.

The intercept here is one published by Glenn Everhart in 1992 as a

candidate standard. It works by replacing the victim DDT with its own and

allows the intercept to be removed completely with a single store of a

longword. It further allows the intercept to be intercepted, and

interception and removal can occur in any order. The reason for proposing

this scheme was precisely that interception and removal become order

independent, so that any third parties using the intercept can freely use

it without risk of crashes due to pool reuse, as occurs where the

interception code is not standardized and an entry is intercepted by

intercept A, then intercept B, and then intercept A is removed. If this

scheme is used, removal of intercept A results in the chain being

reconstituted so that intercept B no longer relies on intercept A's

presence. (In passing, it should be noted that switching has always been

on fairly idle systems, so there may be some cache invalidation needed to

perform such switches safely during load. Setup is done in "safe order"

though, and this intercept is expected to be used where normally it is

not needed to remove it.)

Installation is accomplished via VMSINSTAL. It asks whether to compile

from sources or just link prebuilt object files. Best advice is to

compile if possible. This needs the MACRO assembler on VAX, or the

MACRO32 compiler on alpha; the sources for VAX will work on VMS from

version 5.5 on, and the AXP sources will work from version 7.0 on.

Once you install the kit, you must run the procedure

sys$manager:EA_SETUP.com

which will give a manu which allows the option of doing error avoidance

on all mounted disks or of removing a disk from a previously selected

configuration. If you do not select extra error recovery for all mounted

disks (and you generally will not), the system allows you to go on to

select which disk or disks to do extra error recovery on. You select

disks from a menu until done, then finish.

This produces a file

SYS$MANAGER:EA_START.COM

which should be run at boot to enable the error reduction.

Should you want to turn off this feature for a disk, see in EA_START.COM

what the JGAn: unit number is that is associated with the disk and use

the command

JGCTL/DEASSIGN JGAn: disk:

(This assumes you have cmkrnl privileges). This will restore disk

operation to exactly its normal configuration.